WINDOWS/DOOR MONITORING (log OPEN/CLOSED status)

GOAL for IT: Program the ESP to get the data into the InfluxDB and afterwards evaluation via Grafana.

Quick summary

Assemble the hardware (Wemos D1 mini, sensor), flash NAMF firmware, configure Wi-Fi & InfluxDB in the device web UI, then verify data arrives in InfluxDB and Grafana.

1) What the main components are and what they do

- Wemos D1 mini - ESP-WROOM-32 or other — tiny microcontroller with Wi-Fi. Runs the NAMF firmware.

- Magnetic Door Sensor: Công Tắc Cảm Biến Từ Tính Gắn Cửa Ra Vào Cảnh Báo An Ninh (Shopee).

- Soldering sensor to ESP module for programming. (set up logic level <=> 1= closed (low level = 0V) / 0=open (high level =3.3V))

- USB cable (data) for flashing/power.

- CentOS server running InfluxDB v1 + Grafana v9.2.10 — receives time series and visualizes it (server IP shown in report).

FLASHING the device:

Sensor uses a tiny computer called an ESP32 (Wemos D1 mini / NodeMCU).
-> This chip needs a program to run — the NAMF firmware (using PlatformIO).

Flashing = copying that compiled program into the memory (flash storage) of the ESP so it can run it. = replacing whatever software is on the chip now with your newly built firmware.

- The device must run the NAMF firmware so it can:

✔️ Read temperature/humidity from sensors
✔️ Connect to Wi-Fi
✔️ Send data to your InfluxDB server
✔️ Provide a local web UI for configuration
✔️ Publish values for Grafana to visualize

- What happens immediately after flashing?

✔️ The ESP restarts
✔️ Your new firmware begins running
✔️ It prints logs you can view via Serial Monitor
✔️ It tries to connect to Wi-Fi
✔️ Once connected, it starts sending sensor data

We use PlatformIO to send the firmware binary over USB to the ESP chip: 
pio run -e lang_en -t upload --upload-port COM3

USB / COM port – physical serial connections between your PC and the ESP32. Used for uploading firmware (flashing) and for reading boot/log messages (Use serial monitor software). Examples: COM3 on Windows, /dev/ttyUSB0 on Linux.

Network ports — TCP ports used over the IP network, e.g. 8086 is InfluxDB write port, 3000 is Grafana, 80/443 are HTTP/HTTPS. These are unrelated to the USB COM port.

NOTE: We flash and monitor the device over the COM port, but the device sends sensor data over the network to InfluxDB (port 8086).

1. How to connect firmware NAMF via Wifi and choose port.

2. How to configure it to transfer the sensor data read by firmware to Influx DB.

3. From Inlux DB, connect to Grafana via which IP address?

4.From the MES system, start a new process which runs the key regulations.

ESP after flashing firmware – can auto receive sensor data and push to influx. However, need some local server/cloud to host InfluxDB + Grafana. (CentOS Stream 8)

UPDATE:

PREVIOUS SENSOR + HUMIDITY PROJECT:

- Trong InfluxDB database: _internal (logs) and sensor

- Cmd: curl -G "http://172.25.1.15:8086/query?db=sensor" --data-urlencode "q=SELECT * FROM sensor LIMIT 100"

- Cmd: ssh root@172.25.1.15 - Pass: Company001#VN@2023 - Có URL để quản lý CentOS 8 server. (172.25.1.15:9090)

- Browser:

http://172.25.1.15:8086/query?q=SHOW+DATABASES

http://172.25.1.15:8086/query?q=SHOW+MEASUREMENTS+ON+sensor

- Cách nào để truy cập vào CentOS server để xem con InfluxDB nó được cài đặt thế nào.

- CentOS server do not have a "Desktop", just a box of code. -> No RDP, just Putty to enter. (but likely already locked by firewall for security). Already can log in through ssh root@172.25.1.15.

Questions:

- What platforms we should use (InfluxDB Cloud database or set up local DB server) - Khả năng phải setup local DB server vì influx cloud là bản trả phí.

- How to send data from sensor to Influx through Wifi. (currently have to plug sensor to laptop)

- Then, add DB (including API Key) to Grafana and start visualizing as normal.

Current progress: Wait for RnD to to finish sending data from sensor to Influx through Wifi.

--> Then, only need to add InfluxDB Cloud to Grafana and create visualizations + set alert rules.

Update:

Change plan -> Since InfluxDB Cloud is paid version, switch to old InfluxDB v1 on CentOS to store data, then together find ways to connect/push data from sensors to InfluxDB v1. Once data is on Influx, then connect to Grafana and start visualizing

Update 2/3/2026:

Upload sketch C++ on Arduino, plug USB cable (mounted from ESP module + magnetic sensors) into PC, run the sketch and it returns output on the Serial Monitor console.

Define wifi, influx_host (172.25.1.15), influx_port and influx_db in the .ino file. Define a line (

String url = String("http://") + INFLUX_HOST + ":" + String(INFLUX_PORT) + "/write?db=" + INFLUX_DB;

)
to run.

Bảng mạch ESP8266 của R&D chỉ cần upload arduino sketch lên là nó lưu trên mạch, sau chỉ cần cắm nguồn vào là tự chạy không cần kết nối với laptop

UPDATE: Think about installing and using InfluxDB 3 Core on CentOS Stream 8 server (since it's free and open-source, and up-to-date)

- SSH to server 172.25.1.15: ssh root@172.25.1.15

  - Enter user root and pass: Company001#VN@2023

- Dùng curl install influxDB 3 về server

- Tạo folder influxdata: mkdir influxdata

- Start server: influxdb3 serve --http-bind 0.0.0.0:8181 --data-dir ./influxdata --node-id rrc-node-01

- If failed, cannot bind address, make sure kill all background processes to release port: sudo killall -9 influxdb3

- Check if port is empty and ready to use: sudo ss -tulpen | grep 8181

- Leave server running on 1 terminal, open another terminal to create tokens.

- Create token admin before accessing InfluxDB3 on ip 172.25.1.15:8181:

  - Type influxdb3 create token --admin on a terminal

Token: apiv3_eq3btQrN4edb6VAF23aU-lSJeQHgmuL1FTcBSNfQ_XqO1xrGBZN7JdX1drnsyBscs_nmMOvCQfesoaCVifYPLQ

HTTP Requests Header: Authorization: Bearer apiv3_eq3btQrN4edb6VAF23aU-lSJeQHgmuL1FTcBSNfQ_XqO1xrGBZN7JdX1drnsyBscs_nmMOvCQfesoaCVifYPLQ

- After having token, still leave server running on port 8181, and open another terminal, run this command to access database:
curl -X POST http://172.25.1.15:8181/api/v3/query_influxql -H "Authorization: Bearer [TOKEN]" -H "Content-Type: application/json" -d '{"q": "SHOW DATABASES"}'

- Trick to not type long command: Convert terminal into verified environment:

  - Open config file: nano ~/.bashrc

  - Paste these lines at the end:

export INFLUX_TOKEN="apiv3_eq3btQrN..."

export INFLUX_HOST="http://172.25.1.15:8181"

# Create shortcut

alias exec='curl -s -X POST $INFLUXDB3_HOST_URL/api/v3/query_influxql -H "Authorization: Bearer $INFLUXDB3_AUTH_TOKEN" -H "Content-Type: application/json" -d'

- Save and activate: source ~/.bashrc

- --> Now, only need to type: exec '{"q": "SHOW DATABASES"}'

- Another way to query from influxdb3 databases:

  - influxdb3 show databases --host http://172.25.1.15:8181 --token 'apiv3_eq3btQrN4edb6VAF23aU-lSJeQHgmuL1FTcBSNfQ_XqO1xrGBZN7JdX1drnsyBscs_nmMOvCQfesoaCVifYPLQ'

  - If do not want to specify flags --host and --token in the same command, export variables like below:

    - export INFLUXDB3_HOST_URL='http://YOUR_INFLUX_HOST:8181'

    - export INFLUXDB3_AUTH_TOKEN='YOUR_ADMIN_TOKEN'

    - Then, run command influxdb3 show databases as normal. (And other commands referenced by influxdb3 -help)

- Create a background systemd wrapper service so that influxDB 3 auto runs in background whenever server restarts:

  - Find the path of your binary: which influxdb3 -> Output: /root/.influxdb/influxdb3

  - Create a service file: sudo nano /etc/systemd/system/influxdb3-wrapper.service

  - Paste:

[Unit]

Description=InfluxDB3 Server Wrapper

After=network.target

[Service]

Type=simple

ExecStart=/root/.influxdb/influxdb3 serve --http-bind 0.0.0.0:8181 --data-dir /root/influxdata --node-id rrc-node-01

Restart=always

[Install]

WantedBy=multi-user.target

- Enable the service:

sudo systemctl daemon-reload    (Reload systemd)

sudo systemctl start influxdb3-wrapper  (Start service)

sudo systemctl enable influxdb3-wrapper  (Enable at boot)

- Verify status: systemctl status influxdb3-wrapper

- See logs: journalctl -u influxdb3-wrapper -f

- InfluxDB3 CLI useful commands:

  - influxdb3 show databases

  - Influxdb3 create rrcvn_monitoring

influxdb3 delete table --database rrcvn_monitoring --host http://172.25.1.15:8181 --token apiv3_eq3btQrN4edb6VAF23aU-lSJeQHgmuL1FTcBSNfQ_XqO1xrGBZN7JdX1drnsyBs

cs_nmMOvCQfesoaCVifYPLQ status

- influxdb3 write --database rrcvn_monitoring "status,hostname=TEST,location=Lab state=1 1772442758972100147"

- influxdb3 query "SELECT * FROM status" --database rrcvn_monitoring

- influxdb3 query "SHOW TABLES" --database rrcvn_monitoring

- Check how much disks InfluxDB3 is using: du -sh /root/influxdata or du -h /root/influxdata --max-depth=1

- Check maximum disks available: df -h /

- Check how much disk InfluxDB v1 is using: du -h /var/lib/influxdb --max-depth=1